;============================================================

;   Turn Signal Controller (8051 Version)

;   Using C8051F120-style logic structure

;============================================================

; Capacitive Touch Turn Signal Controller

; C8051F120

$NOMOD51

$include (c8051f120.inc)

; -------- OUTPUTS on Port 2 (ACTIVE-LOW LEDs) --------

LEFT_4  EQU P2.0

LEFT_3  EQU P2.1

LEFT_2  EQU P2.2

LEFT_1  EQU P2.3

RIGHT_1 EQU P2.4

RIGHT_2 EQU P2.5

RIGHT_3 EQU P2.6

RIGHT_4 EQU P2.7

; -------- INPUTS on Port 1 (ACTIVE-LOW SWITCHES) --------

HAZ_SW   BIT P1.1        ; Blink all / hazard

RIGHT_SW BIT P1.2        ; Right indicator

LEFT_SW  BIT P1.4        ; Left indicator

; -------- Hazard blink state flag in RAM --------

HAZ_STATE EQU 20H        ; bit-addressable RAM byte

HAZ_FLAG  BIT 20H.0      ; 0 = LEDs off, 1 = LEDs on

ORG 00H

LJMP Main

ORG 30H

; =========================================

Main:

ACALL INIT

MAIN_LOOP:

MOV P2, #0FFH          ; ensure all LEDs OFF at top of every loop (active-low)

JB HAZ_SW,   HAZARD_MODE   ; switch active-low ? JB means "if 1" (not pressed)

JB RIGHT_SW, RIGHT_MODE

JB LEFT_SW,  LEFT_MODE

; Nothing touched — LEDs stay OFF

SJMP MAIN_LOOP

; =========================================

; Hazard Mode: blink ALL LEDs together using a toggle flag

HAZARD_MODE:

MOV SFRPAGE, #CONFIG_PAGE

JB HAZ_FLAG, HAZ_ON     ; if flag set, turn them ON this cycle

HAZ_OFF:

MOV P2, #0FFH           ; all LEDs OFF (active-low)

SETB HAZ_FLAG           ; next call will turn ON

ACALL WAIT

SJMP MAIN_LOOP

HAZ_ON:

MOV P2, #00H            ; all LEDs ON (active-low)

CLR HAZ_FLAG            ; next call will turn OFF

ACALL WAIT

SJMP MAIN_LOOP

; =========================================

; Right Turn Sequence (active-low)

RIGHT_MODE:

MOV SFRPAGE, #CONFIG_PAGE

CLR HAZ_FLAG            ; reset hazard state

MOV P2, #0FFH           ; start clean — all OFF

CLR RIGHT_1             ; LED 1 ON, rest OFF

ACALL WAIT

CLR RIGHT_2

ACALL WAIT

CLR RIGHT_3

ACALL WAIT

CLR RIGHT_4

ACALL WAIT

MOV P2, #0FFH           ; all LEDs OFF before returning

ACALL WAIT

SJMP MAIN_LOOP

; =========================================

; Left Turn Sequence (active-low)

LEFT_MODE:

MOV SFRPAGE, #CONFIG_PAGE

CLR HAZ_FLAG            ; reset hazard state

MOV P2, #0FFH           ; start clean — all OFF

CLR LEFT_1

ACALL WAIT

CLR LEFT_2

ACALL WAIT

CLR LEFT_3

ACALL WAIT

CLR LEFT_4

ACALL WAIT

MOV P2, #0FFH           ; all LEDs OFF before returning

ACALL WAIT

SJMP MAIN_LOOP

; =========================================

; Delay Routine

WAIT:

MOV R4, #05

L3: MOV R5, #0FFH

L1: MOV R6, #0FFH

L2: DJNZ R6, L2

DJNZ R5, L1

DJNZ R4, L3

RET

; =========================================

; Initialization

INIT:

; Disable watchdog

MOV WDTCN, #0DEH

MOV WDTCN, #0ADH

MOV SFRPAGE, #CONFIG_PAGE

MOV XBR2, #40H          ; Enable crossbar

; Port 2 = LED outputs (push-pull)

ORL P2MDOUT, #0FFH

MOV P2, #0FFH           ; ALL LEDs OFF (active-low)

; Port 1 = switch inputs with pull-ups

MOV P1MDOUT, #00H

MOV P1, #0FFH

CLR HAZ_FLAG            ; initialise hazard blink state to OFF

RET

END